| Product size | 4.3-inch | Type | a-Si TFT-LCD, LCM |
| --- | --- | --- | --- |
| Pixel Format | 480(RGB)×272 [WQVGA] 128PPI | Configuration | RGB Vertical Stripe |
| Active Area | 95.04(H)×53.856(V) mm | Outline Dim. | 105.5(H)×67.2(V) mm |
| Bezel Area | 98.7(H)×57.5(V) mm | Treatment | Antiglare |
| Luminance | 500 cd/m² (Typ.) | Contrast Ratio | 500:1 (Typ.) (TM) |
| View Direction | 6 o'clock | Response Time | 10/15 (Typ.)(Tr/Td) |
| Viewing Angle | 70/70/50/70 (Typ.)(CR≥10) | Operating Mode | TN, Normally White, Transmissive |
| Support Color | 16.7M 50% NTSC | Light Source | WLED [8S1P] , 20K hours , W/O Driver |
| Weight | 59.0g (Typ.) | Designed For | Automotive Display Portable Navigation MP3 PMP |
| Frequency | - | Touch Panel | 4-wire Resistive Touch |
| Interface Type | Parallel RGB (1 ch, 8-bit) , 40 pins FPC |  |  |
| Power Supply | 3.3V (Typ.) |  |  |
| Environment | Operating Temperature: -20 ~ 70 °C ; Storage Temperature: -30 ~ 80 °C |  |  |